Instructions

Asset ID (will be automatically populated in Tool) – The asset ID should be a unique identifier for each asset.  This ID number can be anything, as long as it is unique.  The Asset ID allows for easier asset identification, allows systems to search and query data more effectively, and makes valuable data more readily available.  The Asset IDs should: be logical, be consistent, be unique, avoid duplications, allow room for growth, allow for drilling down by using a hierarchical structure.

For Example:

WMPSPMP1 = Water System Main Street Pump Station Pump 1

W = Water System

MPS = Main Street Pump Station

PMP = Pump

1 = Number 1 Pump

Asset Category = Groups of asset types

  - Enter the Asset Category from the available dropdown menu or enter a custom asset category if necessary. Note that if you select one of the pre-set categories and asset types, the tool will provide an “Estimated Useful Life” for each asset. If a custom category is entered, you will need to manually enter Estimated Useful Life as described below. a. Selection of Asset Category will determine which Asset Types appear in the dropdown menu in Column B.  Predetermined Asset Categories include:

Pump

Hydrant

Meter

Pipe

Source

Storage

Analytical Equipment

Treatment

Valve

Control System

Safety Equipment

Other

Asset Type

Once you have selected the asset Category the program automatically adjusts the options for asset type to only those options that are related to that asset category.  For instance if you add an asset that is a hydrant, the asset category is hydrant and the asset type dropdown box then gives you options as to what type of hydrant it is.

There are similar options for each of the asset categories with respect to the asset types for each category.

Meter

Master

Well

Source

Residential

Commercial

Other

Pipe

Polyvinyl Chloride (PVC)

Ductile Iron (DI)

Steel

Cast Iron

Concrete

Asbestos Cement (AC)

Service Line

Lead Service Line

Other

Source

Well

Intake Structure

Consecutive Connection

Springs

Reservoir

Other

Storage

Bolted, Welded, or Glass Fused Steel Tank

Concrete Tank

Plastic or Polymer Tank

Fiberglass Tank/Structure

Earthen Basin or Reservoir

Hydropneumatic/Captive Air Tank

Other

Analytical Equipment

pH Meter

Chlorine Monitor

Other Remote Sensor

Treatment

Sedimentation

Filtration

Chlorination

Ozonation

Ultraviolet Disinfection

Ion Exchange or Other Media-type Treatment

Solids Handling

Fluoride

Corrosion Control

Other

Valve

Gate

Ball

Butterfly

Pressure Relief (PRV)

Air Release (ARV)

Air Vacuum

Check

Altitude

Control System

Telemetry

SCADA

Variable Frequency Drive

Motor Control Center

Other Programmable Logic Control (PLC)

Other Power Supply

Other Communication System

Safety Equipment

Add details in Asset Description

Other

Building

Vehicle

Computer

Electrical

Lab Equipment

Generator

Tools

Other